# Multi-Modal Wi-Fi Sensing
Detailed Architecture & Methodology

This document is a structured technical guide derived from the survey 'A Short Overview of Multi-Modal Wi-Fi Sensing'. It consolidates the architectures, methodologies, learning paradigms, and implementation patterns discussed across the surveyed literature into a single reference.

## 1. Introduction

Multi-modal Wi-Fi sensing enhances conventional CSI/RSSI sensing by combining Wi-Fi with stronger sensing modalities such as cameras, radar, LiDAR, IMU, Bluetooth, ZigBee and depth sensors. The goal is to improve robustness, domain generalization, sensing accuracy and automatic dataset generation.

## 2. Background

Explain CSI and RSSI. CSI is represented by H=|H|e^(j∠H), containing amplitude and phase for every OFDM subcarrier. The received signal follows Y = HX + N. CSI is decomposed into static and dynamic components, enabling extraction of human motion while suppressing the static environment.

## 3. Overall Taxonomy

The survey groups methods into:
• Multi-modal Fusion
   - Input Fusion
   - Feature Fusion
• Multi-modal Enhanced Training
   - Cross-modal Knowledge Distillation
   - Vision-based Label Generation
• Hybrid approaches combining fusion and knowledge distillation.

## 4. Input Fusion Architecture

Pipeline:
Sensor Acquisition
→ Pre-processing
→ Concatenate modalities
→ Shared Encoder (ViT/GNN)
→ Cross-modal Attention
→ Task Head
Typical applications include localization and human activity recognition.

## 5. Feature Fusion Architecture

Separate encoders process Wi-Fi and auxiliary modalities independently.
WiFi Encoder → Feature
Vision/Radar Encoder → Feature
Cross Attention / Transformer / Feature Concatenation
→ Fusion Layer
→ Prediction.
This design handles heterogeneous data better than raw-input fusion.

## 6. Cross-modal Knowledge Distillation

Teacher: Vision/Radar/RTT
Student: Wi-Fi CSI model
Teacher produces soft targets (dark knowledge).
Student optimizes soft loss and hard loss simultaneously using cross-entropy or MSE.
Benefits include improved generalization and reduced labeling requirements.

## 7. Vision-based Label Generation

Camera and Wi-Fi collect synchronized data.
YOLO/OpenPose/Mask R-CNN automatically generate labels.
Generated labels supervise CSI networks, eliminating manual annotation and enabling scalable dataset creation.

## 8. Feature Alignment

CLIP-style contrastive learning maps Wi-Fi and vision into a shared embedding space.
Modal alignment encourages invariant representations across heterogeneous sensors and improves one-shot or cross-domain adaptation.

## 9. Representative Architectures

SCL: Graph Neural Network with cross-modal attention.
MaskFi: Vision Transformer with masked pre-training.
HDANet: Dual-attention fusion for crowd counting.
ViFi-ReID: CLIP-assisted person re-identification.
X-Fi: Cross-modal Transformer foundation model with missing-modality robustness.
Babel: Prototype alignment with expandable modalities.
WiFitness/WiMix/Wivi-Uf: Wi-Fi + Vision HAR using attention-based fusion.

## 10. Mathematical Components

Key equations:
Y = HX + N
H = |H|e^(j∠H)
Cross-Attention = softmax(QKᵀ/√dk)V
Knowledge Distillation losses:
Cross-entropy soft loss
Embedding MSE loss
CLIP contrastive alignment objective.

## 11. Complete End-to-End Methodology

Acquire synchronized multimodal data → preprocess each modality → encode each stream → perform attention-based fusion or knowledge transfer → align latent representations → train task-specific prediction head → evaluate on cross-domain environments.

## 12. Challenges

Environmental sensitivity, limited labeled datasets, heterogeneous hardware, temporal synchronization, modality alignment, lack of open datasets and reproducibility.

## 13. Future Directions

Foundation models for Wi-Fi sensing, self-supervised learning, LLM-inspired pretraining, Wi-Fi 8 CSI, federated learning, edge AI, cross-domain adaptation and heterogeneous receiver support.

## 14. SilentSense Mapping

Training:
ESP32 CSI + Camera → Teacher Network → Automatic Labels → CSI Foundation Model.
Deployment:
ESP32 CSI → Feature Encoder → Respiration, fall detection, activity recognition, occupancy, sleep monitoring → Digital Twin → LLM reasoning.

## 15. Conclusion

This survey establishes a unified framework where Wi-Fi becomes a central sensing modality strengthened by complementary sensors through fusion, alignment, distillation and automated supervision. These paradigms provide an effective blueprint for next-generation healthcare and ambient intelligence systems.