# Detection of Presence and Number of Persons by a Wi‑Fi Signal
Detailed Architecture & Methodology

A structured technical guide derived from the research paper on RSSI-based presence detection and people counting using Wi-Fi.

## 1. Research Objective

Develop a practical device-free occupancy sensing system using only RSSI measurements from commodity Wi-Fi. Two tasks are addressed: (1) binary presence detection and (2) counting the number of people without requiring them to carry devices.

## 2. System Architecture

Wi-Fi Source
      ↓
RSSI Detectors (3 for presence / 9 for counting)
      ↓
Continuous RSSI Acquisition
      ↓
20-second Window Segmentation
      ↓
Feature Extraction
      ├── Presence Branch
      │      ├─ Standard Deviation Analysis
      │      ├─ Threshold Decision
      │      └─ Isolation Forest
      └── Counting Branch
             ├─ Time-Series Features (tsfresh)
             ├─ KNN / Decision Tree / Random Forest
             └─ Person Count

## 3. Hardware Architecture

2.4 GHz Wi-Fi transmitter with multiple RSSI detectors. Two presence configurations are studied: source inside the room (M1) and source outside (M2). A larger experiment uses one transmitter and nine detectors distributed throughout a 4 × 4.5 m room.

## 4. Experimental Methodology

Presence detection:
• 3 detectors.
• 20-minute recordings for noise and 1–5 people.
• Random walking and stationary behaviour.
• Repeat experiments on a second day.

People counting:
• 9 detectors.
• 20-minute recordings for 1, 3, 5, 7 and 9 people.
• 10 min stationary + 10 min moving.
• Noise recording collected as reference.

## 5. Data Processing Pipeline

RSSI Collection
→ Split into 20-second windows
→ Compute statistical descriptors
→ Feature extraction
→ Classification.
For counting, each 20-minute recording becomes 60 windows, producing 300 labelled samples.

## 6. Presence Detection Methodology

The key observation is that RSSI standard deviation increases significantly when people are present, while the mean RSSI is not a reliable indicator because reflection and absorption can either increase or decrease signal strength.
Three statistical methods are proposed:
1. Thresholding detector-wise standard deviation.
2. Product of detector standard deviations.
3. Product of correlated detector signals.
A fourth approach uses Isolation Forest trained only on noise (semi-supervised anomaly detection).

## 7. Counting Methodology

Presence statistics alone cannot estimate the number of people. Therefore, time-series features are extracted using tsfresh and used for supervised machine learning.
Cross-validation (k=3) trains and evaluates:
• K-Nearest Neighbours
• Decision Tree
• Random Forest

## 8. Machine Learning Pipeline

RSSI Windows
→ Time-Series Feature Extraction
→ Train/Test Split (3-fold Cross Validation)
→ Model Training
→ Prediction
→ Accuracy Evaluation.
Random Forest achieved the best overall performance.

## 9. Mathematical Basis

RSSI fluctuations are caused by human absorption and reflection of Wi-Fi signals. The system models occupancy as a classification problem using statistical descriptors of RSSI time series rather than CSI phase/amplitude information.

## 10. Results

Presence detection:
• Nearly 100% accuracy when source and detectors are inside the room.
• Reduced performance when the source is outside.
People counting:
• Random Forest ≈99% average accuracy.
• Four detectors already achieve ~98%; five detectors ~99%, showing diminishing returns beyond five sensors.

## 11. Strengths and Limitations

Strengths:
• Commodity hardware.
• No wearable devices.
• Simple RSSI processing.
• Excellent accuracy.

Limitations:
• Sensitive to sensor placement.
• Daily calibration recommended.
• Less robust when transmitter is outside the monitored room.
• More complex environments may require additional detectors.

## 12. Adaptation to SilentSense

The methodology can serve as a lightweight occupancy layer:
ESP32 RSSI/CSI → Windowing → Feature Extraction → Occupancy Detection → Person Counting → Higher-level CSI analytics (respiration, falls, activity, digital twin). RSSI provides a low-cost occupancy estimate, while CSI can refine physiological sensing.