# WiEat Architecture and Methodology

Based on the paper 'WiEat: Fine-grained Device-free Eating Monitoring Leveraging Wi-Fi Signals'.

## 1. Overall Architecture

WiFi CSI Collection
→ Data Preprocessing (Calibration, Outlier Removal, Band-pass Filter)
→ Spectrogram-based Activity Segmentation
→ PCA + K-Means (Eating vs Non-Eating)
→ Feature Extraction
→ SVM Classification
→ Probability-based Soft Decision
→ Minute Motion Reconstruction
→ APSD-based Chewing Detection
→ Threshold-based Swallow Detection
→ Final Eating Monitoring Results

## 2. CSI Collection

Collect Channel State Information (CSI) amplitude and phase from 30 OFDM subcarriers using WiFi transmitter/receiver.

## 3. Data Preprocessing

- Remove outliers using IQR.
- Apply band-pass filter.
- Remove ambient RF noise and multipath interference.

## 4. Activity Segmentation

Generate spectrogram, compute Cumulative Power Spectral Density (CPSD) and Cumulative Short Time Energy (CSTE) to detect activity boundaries.

## 5. Eating Activity Detection

Reduce dimensions with PCA and cluster activities using K-Means into Eating and Non-Eating.

## 6. Eating Motion Classification

Extract 14 handcrafted time/frequency-domain features from each of 30 subcarriers. Train Linear SVM to classify Fork, Spoon, Knife+Fork, or Hand.

## 7. Probability-based Soft Decision

Fuse weighted probabilities from every subcarrier instead of majority voting. Higher-variance subcarriers receive higher weights.

## 8. Minute Motion Reconstruction

Use a 250 ms sliding window, select stable subcarriers, reconstruct a representative CSI signal to amplify chewing/swallowing.

## 9. Chewing Detection

Compute FFT and Accumulated Power Spectral Density (APSD). Dominant peak (0.8–3 Hz) estimates chewing frequency.

## 10. Swallow Detection

Differentiate swallowing using amplitude and peak-to-valley duration thresholds.

## 11. Final Outputs

Detect eating, identify utensil, estimate chew count, swallow count, and infer dietary behavior.

## 12. Key Advantages

Hierarchical signal processing pipeline requiring no deep learning, robust to noise, interpretable, and suitable for adaptation to WiFi sensing applications.